Step 16 - Test steering control - old - used before mounting sensor on motor

Assumptions

- Motor is mounted, but not powered

- Angle sensor is mounted, connected to the Teensy which is connected to the RPi and basic testing has been conducted

- The sensor value for hard right and hard left is known

To be completed in this step:

- 3D print the holder for the motor controller and Teensy

- Connect 12V power to the motor controller from battery

- Connect the motor controller (GND, Direction and PWR level) to the Teensy

- Fasten the motor controller and Teensy to the motor mount

- Connect the motor controller power leads to the motor

- Run sample program that reads steering sensor

  - Connect USB drive

    - Make cable long enough to connect

    - Route cable

    - See which devices are currently connected $ ls /dev/ttyACM*

      - /dev/ttyACM0  /dev/ttyACM1  /dev/ttyACM2  /dev/ttyACM3

  - Create a UDEV rule for the front_angle Teensy

- Create a UDEV rule for the front_angle Teensy

  - Background: https://arduino.stackexchange.com/questions/3680/in-linux-how-to-identify-multiple-arduinos-connected-over-usb; http://www.googlinux.com/clearing-dmesg-logs/index.html

  - Logon to the RPI; Confirm the teensy is able to be seen by RPi.  $ ls /dev/ttyACM*, then connect front angle sensor, $ ls /dev/ttyACM*

  - Unplug / turn off power to any USB devices

  - Save the dmesg log. $ dmesg > dmesg-`date +%d%m%Y`.log

  - Clear the log. $ sudo dmesg -c

  - Plug in the Teensy to be used.  Remember this has a unique serial number.

  - List the dmesg $ dmesg

  - Results:

ubuntu@ubiquityrobot:~$ dmesg [118616.881363] usb 1-1.3.2: new full-speed USB device number 45 using dwc_otg [118617.014192] usb 1-1.3.2: New USB device found, idVendor=16c0, idProduct=0483 [118617.014207] usb 1-1.3.2: New USB device strings: Mfr=1, Product=2, SerialNumber=3 [118617.014215] usb 1-1.3.2: Product: USB Serial [118617.014223] usb 1-1.3.2: Manufacturer: Teensyduino [118617.014230] usb 1-1.3.2: SerialNumber: 5588760 [118617.015401] cdc_acm 1-1.3.2:1.0: ttyACM0: USB ACM device

- Setup udev rule mapped to “steer_motor ” because later you will want to update the setting: “upload port = /dev/steer_motor ” in the platformio.ini file.  Background reading: https://linuxconfig.org/tutorial-on-how-to-write-basic-udev-rules-in-linux

- Edit and update this line, then copy it because it will be easier to paste into the editor.

- SUBSYSTEM=="tty", ATTRS{idVendor}=="16c0" , ATTRS{idProduct}=="0483" , ATTRS{serial}=="5588760", SYMLINK+="steer_motor"

- Change permissions so I can use FileZilla and Sublime to edit the udev files

  - $ sudo chmod -R 777 /etc/udev/rules.d/05-serial.rules

- $ sudo edit /etc/udev/rules.d/05-serial.rules

- Move the cursor to the end of the last character in the last line

- Press INSERT; Move the cursor to the end and press enter

- Copy the new line above and paste into the editor by right mouse clicking at the beginning of the new line.

- Press Escape to exit the insert mode

- Save the file by entering “:w” then enter

- Quit using “:q” then enter

- Reboot the RPi for the changes to take effect $ sudo reboot

- $ ls /dev/steer_motor  to confirm the link has been made

- Update PlatformIO environment ini

- Open FileZilla, Update the platformio.ini file in /home/ubuntu; Add section for front_angle

; [env:front_angle] platform = teensy board = teensy31 framework = arduino upload_port = /dev/front_angle

- “board = teensy35” is used for a Teensy 3.5

- Update bash scripts to help PlatformIO compiling

  - Background: There are two bash scripts that have been used for compiling, compile_a.sh and compile_b.sh.  These scripts need to be updated because you have a new microcontroller connected to the USB hub.

  - Connect all the USB cables and run $ grep 16c0 /sys/bus/usb/devices/*/idVendor to list all the Teensy devices connected.

  - For testing, run the scripts without the platformio or roslaunch commands enabled.  I had issues getting permissions correct.

  - If you reboot, to resolve permission issues run:

    - $ sudo chown root:plugdev /sys/bus/usb/drivers/usb/{bind,unbind}

    - $ sudo chmod g+w /sys/bus/usb/drivers/usb/{bind,unbind}

    - See: https://community.platformio.org/t/multiple-boards-upload-port-works-but-crashes-2nd-board/9029/8

ubuntu@ubiquityrobot:~$ grep 16c0 /sys/bus/usb/devices/*/idVendor /sys/bus/usb/devices/1-1.3.1.1/idVendor:16c0 (joystick) /sys/bus/usb/devices/1-1.3.1.2/idVendor:16c0 (left speed) /sys/bus/usb/devices/1-1.3.1.3/idVendor:16c0 (transmission) /sys/bus/usb/devices/1-1.3.1.4/idVendor:16c0 (front_angle) /sys/bus/usb/devices/1-1.3.2/idVendor:16c0 (steer_motor) (NOTE: Put the latest device IDs in the file “Documenting the Teensy device ID’s”)

File: compile_a.sh #!/bin/bash # this script to "turn off" the USB ports and to start the compile process # # Comment out the device you will be programming # # (1 of 2) Comment OUT the device you WILL be programming or devices not connected # #echo -n "1-1.3.1.1" > /sys/bus/usb/drivers/usb/unbind # diy joystick echo -n "1-1.3.1.2" > /sys/bus/usb/drivers/usb/unbind # left speed echo -n "1-1.3.1.3" > /sys/bus/usb/drivers/usb/unbind # transmission echo -n "1-1.3.1.4" > /sys/bus/usb/drivers/usb/unbind # front_angle #echo -n "1-1.3.2" > /sys/bus/usb/drivers/usb/unbind # steer_motor # # (2 of 2) UN-COMMENT the line for the platformio environment you WANT TO COMPILE # # Platformio environments include: [env:joystick], [env:left_speed], [env:transmission], [env:front_angle], [env:steer_motor] #sudo platformio run -t upload -e front_angle # for [env:front_angle] sudo platformio run -t upload -e steer_motor # for [env:steer_motor] #sudo platformio run -t upload -e transmission # for [env:transmission]

File: compile_b.sh #!/bin/bash # this script to "turn on" the USB ports after the compile process is complete # # Comment OUT the board that IS being programmed. It will not need the "bind" statement. All other connected devices should be active. # #echo -n "1-1.3.1.1" > /sys/bus/usb/drivers/usb/bind # joystick echo -n "1-1.3.1.2" > /sys/bus/usb/drivers/usb/bind # left speed echo -n "1-1.3.1.3" > /sys/bus/usb/drivers/usb/bind # transmission echo -n "1-1.3.1.4" > /sys/bus/usb/drivers/usb/bind # front_angle #echo -n "1-1.3.2" > /sys/bus/usb/drivers/usb/bind # steer_motor # # start the launch file that will initiate the Teensy's you want to test cd /home/ubuntu/catkin_ws/src roslaunch teensy_launch.launch #roslaunch steer_test_launch.launch #roslaunch transmission_test_launch.launch

To test:

- $ grep 16c0 /sys/bus/usb/devices/*/idVendor

- $ ls /dev/ttyACM*

- If you have code to compile and multiple Teensy’s are connected and active, run $ bash compile_a.sh  then $ ls /dev/ttyACM*

- Alternatively if the USB ports are still unbound (i.e. only one Teensy active), run $ sudo platformio run -t upload -e steer_motor

- Once compile is clean and you want to test, run $ bash compile_b.sh  then $ ls /dev/ttyACM*

- If you have seen how they appear, disappear and reappear, you can now take out the comment for the PlatformIO statement in compile_a.sh

  - If USB ports are still unbound, run $ sudo platformio run -t upload -e steer_motor

- Test compiling and running blink.cpp

- Update launch file - steer_test_launch.launch to only launch the angle and motor controller teensy.

steer_test_launch.launch

<launch> <?ignore - used to comment out lines <node ns="transmission_teensy" name="transmission_cntrl" pkg="rosserial_python" type="serial_node.py" args="/dev/transmission_control" output="screen" /> <node ns="potentiometer_teensy" name="potentiometer_output" pkg="rosserial_python" type="serial_node.py" args="/dev/potentiometer_control" output="screen" /> <node ns="left_speed_teensy" name="left_speed" pkg="rosserial_python" type="serial_node.py" args="/dev/odom_left" output="screen" /> ?> <node ns="front_angle_teensy" name="front_angle" pkg="rosserial_python" type="serial_node.py" args="/dev/front_angle" output="screen" /> <node ns="steer_motor_teensy" name="steer_motor" pkg="rosserial_python" type="serial_node.py" args="/dev/steer_motor" output="screen" /> </launch>

- Test program

  - Once you have a complete compile and before launching the programs

    - Check power to motor

    - Ensure the work area is clear; Double check the steering rods and there are no obstacles (e.g. wires) in the way

    - Remind yourself where to disconnect power should you need to do so

    - On RPi,

      - $ ls /dev  # you should only see one teensy (i.e. steer_motor)

      - Run $ bash compile_b.sh

      - Optionally if the bind statements have already been run:

        - $ cd /home/ubuntu/catkin_ws/src

        - $ roslaunch steer_test_launch.launch

  - On Laptop

    - Ctrl+Alt+T, $ rqt

    - Organize rqt screens to monitor steering PWM, cmd_vel/angular/x and front_angle_klm/data.  Use “dock” icon to move screens to desired position

    - Connecting the joystick - See Step 4 - Install ROS-Ag lawn tractor simulator section Test drive using gamepad for background  and to confirm port identifier.

    - Start joy_input to transfer data to cmd_vel:

- Ctrl+Alt+T, $ roslaunch tractor_teleop cmd_vel_mux.launch

- Ctrl+Alt+T, $ roslaunch tractor_teleop drive_teleop.launch joy_dev:=/dev/input/js0

- Note: drive_teleop.launch has as a default: <arg name="joy_dev" default="/dev/input/js0"/>  If your joystick is on js1 you can either change the launch file or adjust the argument.

- Alternatively, the command below has both of the above commands combined

  - $ roslaunch tractor_teleop drive.launch

    - At this point you should be able to press the deadman switch, move the steering joystick and see the target angular.z adjust.

Next Step

Confirm steer straight and left right settings.

- Choose Plugins -> Robot Tools -> Robot Steering

In the rqt topic put /teleop/cmd_vel in the topic field

On the RPi echo the topic and move the slider in rqt to see the topic being published

- Drag the window all the way to the right

- You can now adjust the slide bars and send cmd_vel command using this interface

- An alternative is to run a bash script written for this purpose.

#!/bin/bash timeout 10 rostopic pub -r 10 teleop/cmd_vel geometry_msgs/Twist '{linear: {x: 0.0, y: 0.0, z: 0.0}, angular: {x: 0.0,y: 0.0,z: 0.5}}' timeout 10 rostopic pub -r 10 teleop/cmd_vel geometry_msgs/Twist '{linear: {x: 0.0, y: 0.0, z: 0.0}, angular: {x: 0.0,y: 0.0,z: 0.0}}' timeout 10 rostopic pub -r 10 teleop/cmd_vel geometry_msgs/Twist '{linear: {x: 0.0, y: 0.0, z: 0.0}, angular: {x: 0.0,y: 0.0,z: 0.5}}'

- Plugins -> Visualizations -> Plot will allow you to subscribe and visualize (i.e. plot) rostopics

- Plugins -> Topics -> Topic Monitor lets you drag and drop the topics you are interested in

Testing protocol

| Cmv_vel angular.z | Points to look for |
| --- | --- |
| 0.0 | Is the steering steady and straight; Is the steer effort zero |
| 0.03 to -0.03 | Does the steering react and move to these smaller increments |
| Move from 0 to +/- .5 | Does the steering reach the target and steer effort go to zero? What there overshoot? How long did it take to achieve the target? |
| 1 and -1 | Does the steering reach the target and steer effort go to zero? What there overshoot? How long did it take to achieve the target? |

Example output going from: 0.0 -> -0.5 ->  +0.5 -> -0.7 -> +0.7 -> 0.0

$ timeout 3 rostopic pub -r 10 /cmd_vel geometry_msgs/Twist  '{linear:  {x: 0.0, y: 0.0, z: 0.0}, angular: {x: 0.0,y: 0.0,z: 0.5}}'

$ timeout 3 rostopic pub -r 10 /cmd_vel geometry_msgs/Twist  '{linear:  {x: 0.75, y: 100.0, z: 0.0}, angular: {x: 200.0,y: 0.0,z: 0.0}}'

10/15/19

Using rosparam to change settings

The command <rosparam param="steering">[28900.1, 46300.0, 36500.0, 50.0, 2.0]</rosparam> is located in the teensy_launch.launch and steer_test.launch files.  This sets the steering parameters:

steer_left_max = 28900;

steer_right_max = 46300;

steer_straight = 36500;

max_steer_eff = 50;

steer_Ki = 2;

The command  that will also do the same thing is $ rosparam set /steering "[28900.0, 46300.0, 36500.0, 50.0, 2.0]"

I adjusted the steer straight to be 36800

10/18/19

- $ rosparam set /steering "[28900.0, 46300.0, 38000.0, 70.0, 2.0]"

Once you have the transmission running you can further tune the settings.  For example, in the grass, get the steer_effort adjusted

3/1/20

Import KLM perspectives in rqt

Dragging from right to left with the right mouse button down will slow down the speed of the plot.

Press the up/down arrow pan axis first

Doing the same up and down to zoom in on the preferred range.

| Straight ahead position_klm: 36102 |  |  |
| --- | --- | --- |
| Full left position_klm: 27184 Full right position_klm: 47415 |  |  |

The steering is not making the full rotation; Check variables being passed.

5/17/20

$ timeout 3 rostopic pub -r 10 /cmd_vel geometry_msgs/Twist  '{linear:  {x: 0.0, y: 0.0, z: 0.0}, angular: {x: 0.0,y: 0.0,z: 0.5}}'

No movement at all - This issue was the signal wire to the motor controller had come loose.

$ rosparam set /steering “[27900.0, 45798.0, 36874.0, 60.0, 3.0]”

- $ rosparam set /steering "[27900.0, 45798.0, 36874.0, 60.0, 3.0]"

- $ rosparam set /steering "[23600.0, 44300.0, 33319.0, 60.0, 3.0]"

<rosparam param="steering">[24300.0, 44000.0, 34000.0, 70.0, 3.0]</rosparam>

5/18/20

The wheels banged into a big ant mound and turned the wheels hard one direction.  I need to re-confirm left, right, center.

Current:$ rosparam get /steering

[24300.0, 44000.0, 34000.0, 70.0, 3.0]

Left

Right

- $ rosparam set /steering "[27400.0, 45200.0, 35600.0, 70.0, 3.0]"

0.5, -0.3, 60  - produces a circle about 15’ in diameter

0.5, -0.2, 60 - produces a circle about 17’ in diameter

Positive still produced a right hand turn!

0.5, 0.005, 60

After 30 minutes of testing the center seems to be off which is leading to the inability to turn left.

I need to recenter.: 29209

Full left: 19200

Full right: 42000

42022

- $ rosparam set /steering "[19200.0, 42000.0, 29209.0, 70.0, 3.0]"

Pulling left when going straight

- rosparam set /steering "[19200.0, 42000.0, 30000.0, 70.0, 3.0]"

- rosparam set /steering "[19200.0, 42000.0, 30400.0, 70.0, 3.0]"

Center has become unadjusted again.  It is near 40000!

Just to get the tractor back

- rosparam set /steering "[38200.0, 46000.0, 42000.0, 70.0, 3.0]"

5/22/20

2 rotation potentiometer keeping track of the rotations

Continuous rotation potentiometer

https://www.usdigital.com/products/encoders/absolute/magnetic/MA3