Step 16 - Test steering control

Assumptions

- Transmission control servo is mounted and controlled by a Teensy

- Wheel speed sensor is mounted and controlled by a Teensy

To be completed in this step:

- Program both Teensy’s

- Configure PID settings

- Connect the motor controller power leads to the motor

- Run sample program that reads steering sensor

  - Connect USB drive

    - Make cable long enough to connect

    - Route cable

    - See which devices are currently connected $ ls /dev/ttyACM*

      - /dev/ttyACM0  /dev/ttyACM1  /dev/ttyACM2  /dev/ttyACM3

  - Create a UDEV rule for the front_angle Teensy

- Create a UDEV rule for the front_angle Teensy

  - Background: https://arduino.stackexchange.com/questions/3680/in-linux-how-to-identify-multiple-arduinos-connected-over-usb; http://www.googlinux.com/clearing-dmesg-logs/index.html

  - Logon to the RPI; Confirm the teensy is able to be seen by RPi.  $ ls /dev/ttyACM*, then connect front angle sensor, $ ls /dev/ttyACM*

  - Unplug / turn off power to any USB devices

  - Save the dmesg log. $ dmesg > dmesg-`date +%d%m%Y`.log

  - Clear the log. $ sudo dmesg -c

  - Plug in the Teensy to be used.  Remember this has a unique serial number.

  - List the dmesg $ dmesg

  - Results:

ubuntu@ubiquityrobot:~$ dmesg [118616.881363] usb 1-1.3.2: new full-speed USB device number 45 using dwc_otg [118617.014192] usb 1-1.3.2: New USB device found, idVendor=16c0, idProduct=0483 [118617.014207] usb 1-1.3.2: New USB device strings: Mfr=1, Product=2, SerialNumber=3 [118617.014215] usb 1-1.3.2: Product: USB Serial [118617.014223] usb 1-1.3.2: Manufacturer: Teensyduino [118617.014230] usb 1-1.3.2: SerialNumber: 5588760 [118617.015401] cdc_acm 1-1.3.2:1.0: ttyACM0: USB ACM device

- Setup udev rule mapped to “steer_motor ” because later you will want to update the setting: “upload port = /dev/steer_motor ” in the platformio.ini file.  Background reading: https://linuxconfig.org/tutorial-on-how-to-write-basic-udev-rules-in-linux

- Edit and update this line, then copy it because it will be easier to paste into the editor.

- SUBSYSTEM=="tty", ATTRS{idVendor}=="16c0" , ATTRS{idProduct}=="0483" , ATTRS{serial}=="5588760", SYMLINK+="steer_motor"

- Change permissions so I can use FileZilla and Sublime to edit the udev files

  - $ sudo chmod -R 777 /etc/udev/rules.d/05-serial.rules

- $ sudo edit /etc/udev/rules.d/05-serial.rules

- Move the cursor to the end of the last character in the last line

- Press INSERT; Move the cursor to the end and press enter

- Copy the new line above and paste into the editor by right mouse clicking at the beginning of the new line.

- Press Escape to exit the insert mode

- Save the file by entering “:w” then enter

- Quit using “:q” then enter

- Reboot the RPi for the changes to take effect $ sudo reboot

- $ ls /dev/steer_motor  to confirm the link has been made

- Update PlatformIO environment ini

- Open FileZilla, Update the platformio.ini file in /home/ubuntu; Add section for front_angle

; [env:front_angle] platform = teensy board = teensy31 framework = arduino upload_port = /dev/front_angle

- “board = teensy35” is used for a Teensy 3.5

- Update bash scripts to help PlatformIO compiling

  - Background: There are two bash scripts that have been used for compiling, compile_a.sh and compile_b.sh.  These scripts need to be updated because you have a new microcontroller connected to the USB hub.

  - Connect all the USB cables and run $ grep 16c0 /sys/bus/usb/devices/*/idVendor to list all the Teensy devices connected.

  - For testing, run the scripts without the platformio or roslaunch commands enabled.  I had issues getting permissions correct.

  - To resolve permission issues I ran:

    - $ sudo chown root:plugdev /sys/bus/usb/drivers/usb/{bind,unbind}

    - $ sudo chmod g+w /sys/bus/usb/drivers/usb/{bind,unbind}

    - See: https://community.platformio.org/t/multiple-boards-upload-port-works-but-crashes-2nd-board/9029/8

ubuntu@ubiquityrobot:~$ grep 16c0 /sys/bus/usb/devices/*/idVendor /sys/bus/usb/devices/1-1.3.1.1/idVendor:16c0 (joystick) /sys/bus/usb/devices/1-1.3.1.2/idVendor:16c0 (left speed) /sys/bus/usb/devices/1-1.3.1.3/idVendor:16c0 (transmission) /sys/bus/usb/devices/1-1.3.1.4/idVendor:16c0 (front_angle) /sys/bus/usb/devices/1-1.3.2/idVendor:16c0 (steer_motor)

File: compile_a.sh #!/bin/bash # this script to "turn off" the USB ports and to start the compile process #/sys/bus/usb/devices/1-1.3.1.1/idVendor:16c0 (joystick) #/sys/bus/usb/devices/1-1.3.1.2/idVendor:16c0 (left speed) #/sys/bus/usb/devices/1-1.3.1.3/idVendor:16c0 (transmission) #/sys/bus/usb/devices/1-1.3.1.4/idVendor:16c0 (front_angle) #/sys/bus/usb/devices/1-1.3.2/idVendor:16c0 (steer_motor) # # # Comment out the device you will be programming # echo -n "1-1.3.1.1" > /sys/bus/usb/drivers/usb/unbind echo -n "1-1.3.1.2" > /sys/bus/usb/drivers/usb/unbind echo -n "1-1.3.1.3" > /sys/bus/usb/drivers/usb/unbind echo -n "1-1.3.1.4" > /sys/bus/usb/drivers/usb/unbind #echo -n "1-1.3.2" > /sys/bus/usb/drivers/usb/unbind # (steer_motor) # # Un-comment the line for the platformio enviromnent you want to compile # #sudo platformio run -t upload -e front_angle sudo platformio run -t upload -e steer_motor

File: compile_b.sh #!/bin/bash # this script to "turn on" the USB ports after the compile process is complete #/sys/bus/usb/devices/1-1.3.1.1/idVendor:16c0 (joystick) #/sys/bus/usb/devices/1-1.3.1.2/idVendor:16c0 (left speed) #/sys/bus/usb/devices/1-1.3.1.3/idVendor:16c0 (transmission) #/sys/bus/usb/devices/1-1.3.1.4/idVendor:16c0 (front_angle) #/sys/bus/usb/devices/1-1.3.2/idVendor:16c0 (steer_motor) # echo -n "1-1.3.1.1" > /sys/bus/usb/drivers/usb/bind echo -n "1-1.3.1.2" > /sys/bus/usb/drivers/usb/bind echo -n "1-1.3.1.3" > /sys/bus/usb/drivers/usb/bind echo -n "1-1.3.1.4" > /sys/bus/usb/drivers/usb/bind #echo -n "1-1.3.2" > /sys/bus/usb/drivers/usb/bind # cd /home/ubuntu/catkin_ws/src #roslaunch teensy_launch.launch

To test:

- $ grep 16c0 /sys/bus/usb/devices/*/idVendor

- $ ls /dev/ttyACM*

- $ bash compile_a.sh  then $ ls /dev/ttyACM*

- $ bash compile_b.sh  then $ ls /dev/ttyACM*

- If you have seen how they appear, disappear and reappear, you can now take out the comment for the PlatformIO statement in compile_a.sh

- Test compiling and running blink.cpp

- Update launch file - steer_test_launch.launch

- Test program

  - If USB ports are still unbound, run $ sudo platformio run -t upload -e steer_motor

  - Once you have a complete compile and before launching the programs

    - Check power to motor

    - Ensure the work area is clear; Double check the steering rods and there are no obstacles (e.g. wires) in the way

    - Remind yourself where to disconnect power should you need to do so

    - Run $ bash compile_b.sh

Problem:

- Need to confirm current steer angle is being published and read by test_steer

- This is a good reminder there should be a watchdog process that if the steer angle stops being published then all systems should stop

  - Make sure /front_angle_avg is publishing data

    - Run

    - $ cd /home/ubuntu/catkin_ws/src

    - $ roslaunch steer_test_launch.launch

    - Notes:

      - Angle is reading 49186 - the wheels are turned all the way to the right - since the extreme right is 49419 the angle sensor appears to be working

      - On Ubuntu laptop $ rosrun rqt_plot /front_angle_avg

Useful video explaining setting up publisher/subscriber: https://www.youtube.com/watch?v=2Cmdu6mkxD0

$ rostopic info {topic}

$ rostopic info front_angle_avg

$ rosmsg show {type}

$ rosmsg show std_msgs/Float32

ubuntu@ubiquityrobot:~$ rosmsg show std_msgs/Float32

float32 data

ubuntu@ubiquityrobot:~$

$ rosnode info /steer_motor_teensy/steer_motor

(const nav_msgs::Odometry::ConstPtr& msg) {

|  | Yellow (pin 2) ->DIR (direction, High or Low) RED (pin 3) ->PWM (speed, 0-255) // Set the rotation direction. digitalWrite(2, HIGH); // Turn the motor on at half speed analogWrite(3, 127); |
| --- | --- |
|  | Connect all the wires, but for initial testing do not connect the “A” power from the controller to the motor. I don’t want to run the motor yet, until I’m sure the software is OK. |
|  | - Run test_steer.cpp. I have hardcoded a steet_angle_target of 1042. There is a buffer of 20 positions (1022 - 1062) will produce a ROS_INFO message and the wheels should be straight. - Move the wheels left or right and the LEDs should light up indicating the motor controller being engaged to bring the wheels straight - (HOW DO I CONFIRM THESE ARE NOT BACKWARDS - i.e LEFT is really the left I want and vice versa.) |

Background references:

- https://www.cytron.io/p-30amp-5v-30v-dc-motor-driver  (OEM website)

- https://docs.google.com/document/d/178uDa3dmoG0ZX859rWUOS2Xyafkd8hSsSET5-ZLXMYQ/view  (OEM ref for user manual)

- https://www.instructables.com/lesson/Reversing-a-Motor/  (sample Arduino program)

ros::Subscriber <std_msgs::Float32> sub("/front_angle_avg", get_front_angle);

9/2/19

Problem:

Steering program is not stopping when the error is small

I think I need a compound if statement

9/3/2019

- Changing the signs

9/8/19

Writing program

To read cmd_vel and move steering based on it.

Testing process:

- Plug in joystick on laptop

- Start simple_drive to publish to cmd_vel

- On RPi, start the steer_motor controller

- On laptop, $ roslaunch simple_drive drive_teleop.launch joy_dev:=/deinput/js0

9/9/2019

- Make changes to steer_control_v1.cpp to add publishing target position

- Use FileZilla to move the new source code to /src

- $ ls /dev/ttyACM*

- $ bash compile_a.sh

  - If the compile is not clean $ sudo platformio run -t upload -e steer_motor

- Once compile is clean…. $ bash compile_b.sh

  - If you just need to launch the program:

    - $ cd /home/ubuntu/catkin_ws/src

    - $ roslaunch steer_test_launch.launch

- On laptop, Ctrl+Alt+T, $ rqt

- Choose Plugins -> Robot Tools -> Robot Steering

In the rqt topic put /teleop/cmd_vel in the topic field

On the RPi echo the topic and move the slider in rqt to see the topic being published

- Drag the window all the way to the right

- You can now adjust the slide bars and send cmd_vel command using this interface

- Plugins -> Visualizations -> Plot will allow you to subscribe and visualize (i.e. plot) rostopics

- Plugins -> Topics -> Topic Monitor lets you drag and drop the topics you are interested in

Note

9/14/19

Debugging the target

The Output does not go negative; Found a set limits command

| Current | Target | Steer_err_value | cmd_vel |
| --- | --- | --- | --- |
| 36357 | 36341 | -15 | -0.01 |
| 36357 | 36432 | 75 | -0.02 |

Testing protocol

| Cmv_vel angular.z | Points to look for |
| --- | --- |
| 0.0 | Is the steering steady and straight; Is the steer effort zero |
| 0.03 to -0.03 | Does the steering react and move to these smaller increments |
| Move from 0 to +/- .5 | Does the steering reach the target and steer effort go to zero? What there overshoot? How long did it take to achieve the target? |
| 1 and -1 | Does the steering reach the target and steer effort go to zero? What there overshoot? How long did it take to achieve the target? |

Example output going from: 0.0 -> -0.5 ->  +0.5 -> -0.7 -> +0.7 -> 0.0